- Short manual to the EXCEL-tool: TRUSS

1. Announcements / Preparations

Announcements

This short manual to the EXCEL-based tool TRUSS which accompanies the course in Non-linear FEM shall help to give a first introduction and some hints concerning the possibilities and the handling of the software.

Where to download:

EXCEL-based tool TRUSS can be downloaded at the Moodle page of the course Non-linear FEM or the homepage of the chair of structural analysis under Home/Software/Education/Truss

Content:

The manual is subdivided in three main parts:

- Sections 2-7 explain the basic steps in a simple example (two-bar truss)

- Section 8 presents the different solution algorithms (i.e. path following methods) and section 9 illustrates some special features

- Section 10 and 11 explain how to export & import models.

- Section 12 answers some questions and shall give hints to work around some bugs

If you would like to enhance the tutorial with more hints you are cordially invited.

Preparations

Generally: This tutorial implies the use of Microsoft EXCEL® 2007, 2010 or 2013.

General preparations in Excel:

TRUSS consists of one main file Truss20_20200420.xlsm, which is an EXCEL® file with macros. For this reason you have to enable the use of macros in EXCEL®. There are two ways to do so:

- Standard settings:

By default macros are blocked and EXCEL® and when opening the main file Truss20_20200420.xlsm EXCEL® asks if you would like to activate them. You have to confirm this.

- Allow macros by default:

Main Menu in EXCEL 2013

Then go to Trust Center and click on Trust Center Settings.

Trust Center in EXCEL 2013

Go to Macro Settings and activate the check box Enable all macros.

Trust Center in EXCEL 2013

Remark: This option allows Excel in general the usage of Macros!

Start a calculation

- Download the program from the website of the course NFEM

- Unzip the file to a local directory (e.g. Desktop)

- Open the file Truss20_20200420.xlsm (above of Cell A1, a security warning dialog appears; click on “Options” and mark the buttons “enable this content” in order to activate the macros).

- The file is already filled with the input of the following 2-bar-truss example.

In the following, the principles of how to work with the program will be explained in a short example. If you encounter an error message, have a look at Section 10.

Example:

Task: Analyse the deformation behaviour of the system given above. Follow the primary path using proper path-following methods and determine critical points.

# New Problem

2. Nodes & Boundary Conditions

Start with the nodes-sheet:

# new

- Initialize a new problem by clicking on

- Enter the node numbers and the respective coordinates

- Enter the support conditions for every node:

Every node has 3 degrees of freedom (DOF) u,v,w in the global directions x,y,z  which can be either fixed free. To ensure that a DOF is fixed, this means the deformation in this direction is blocked, enter any string in the appropriate cells (e.g. “support”). In case the DOF should be free leave the cell empty.
If the support consists of a spring, enter the spring’s stiffness in these cells.

# Read

- Read in the entered data by clicking	              (you automatically jump to the Elements-sheet)

# BC

# eqn.#

# displacement

- By clicking on	                     ,	                  or			  , you can switch between the following views in the Nodes-sheet:

| # BC | Boundary conditions (initial view) |
| --- | --- |
| # eqn.# | Equation number of the specific DOF |
| # displacement | displacement of the specific DOF |

3. Elements

- Enter the element number, its beginning (i) and ending node (k)

- Enter the Young’s modulus E and the cross-section A

- F0 would be the pre-stress, which we don’t need for the moment. Just leave it empty (i.e. = 0)

- Choose the element formulation. Use the acronyms explained in the blue box on the right.

# Read

- Read in the entered data by clicking	g	(you jump automatically to the Loads-sheet)

4. Loads

- Define the position of the loads by entering the node-number (column B)

- The components of the load (Fx,Fy,Fz) have to be given in global directions (negative sign  opposite direction to the global axis); per node only one set of loads (Fx,Fy,Fz) can be used.

# 0,01

- Enter the load factor in the red marked cell e.g.		           . All loads will be scaled with the one and only load factor .

# Read

- Read in the entered data by clicking	                 (you automatically jump to the Grafik-sheet)

5. Graphic

# OK

A pop-up menu appears, giving you different options. By confirming with 		you accept the options and come to the Grafik-sheet.

This sheet provides a view of the entered system. Different options and feature for the visualisation can be entered in the menu behind the		  	-button.

# Grafic

You can visualize your system at any time. By clicking on   		          you can specify a certain time step. However, after choosing the time step the graphic still needs to be updated.

# Time step

# Grafic

Note: The graphic is not updated automatically. For an updated graphic click on	  	(you will come to the same pop-up menu as before) and then on

# OK

# Solve

Click  	     (you automatically jump to the System-sheet)

6. Solving the problem

A pop-up menu “solution options” automatically appears, giving three solution algorithms:

- load control

- arc length method

- displacement control

Note: these algorithms and their handling will be explained more in detail in section 7.

This dialog is also available by clicking			  in the System-sheet.

# Sol.Options

in the System-sheet.

- Choose “load_control" as solution algorithm (for this example).

- Check all display-options in the “general parameters” column.

- Choose a solver and set the number of “max. Iteration” to e.g. 80.

# OK

- Confirm with

Now the system-sheet is available:

The diagram depicts the “Load Factor – Displacement” Curve.

The data to be shown in the diagram can be chosen by clicking on

# Plot

In this dialog you can enter and add (or remove) specific curves of the diagram.

# remove

- To remove a line, select it and the click on

To add a line, specify the data plotted in the horizontal and the vertical axis. For displacements (“disp”), select the DOF which should be used. 
You can find the DOF for a certain displacement of a node in the Nodes-sheet by clicking on

# eqn.#

- Do not forget to label the new data curve with a meaningful label.

To add the new data curve click on

# add

Confirm with

# OK

Above the diagram, you can see the equation system with the current stiffness matrix, the load, the residuum (RHS) and displacement for the specific DOFs.

# Solve

Press 		      to solve the equation system and to go one step further on the path of equilibrium. The load-displacement curve will be updated accordingly.

# Rhs

For control: the residuum vector (right hand side vector)		should be approximately equal to zero.

# Loads

To compute the next time step on the path of equilibrium, click on	 	       and enter a new load-factor in the Loads-sheet.

# Read

Confirm with

# Solve

In the System-sheet press                       to solve the new equation system. The load-displacement curve will be updated accordingly.

7. Movie

# Movie

By clicking on 		you come to the Movie-sheet where you can watch the results for each time step.

The view of the system (Grafik-sheet) can be changed by rotating and moving.

Basic functions are:

- Scroll

- Rotation (all axes)

- Scaling factor

- Time steps

8. Solution algorithms

# Sol. options

Clicking on		             in the System-sheet gives options for the solution algorithm:

The goal of all algorithms is to find the next point of equilibrium on the unknown load-displacement curve.

- By using the "load control method" displacements are calculated to satisfy equilibrium to the given load factor.

- The "displacement control method" determines the load factor to satisfy equilibrium for prescribed displacements.

- The "arc length method" creates a scaled tangent vector (predictor) at the end of the existing equilibrium path. The initial point of this vector is the midpoint of a circle, the length is the radius. For the so-defined circle the intersection with the equilibrium path will be evaluated. This intersection is the next point of equilibrium, which is displayed in the plot. In general both, the load factor and the displacements, are changed at the same time by prescribing the predictor.

8.1 Load-control

 Already described in the example above.

8.2 Displacement Control

# Solution options

Click on 		    in the System-sheet in order to get to the following dialog:

- Activate "displacement control" .

- This method requires the prescribed displacement for one DOF.

For our example this could be the vertical displacement of the top node: displacement v of node 2

# OK

- Confirm your input with

# Nodes

- Click on 	           to get to the nodes-sheet. Here you can enter the prescribed displacement for the specified node in the now the red marked cell.

# Read

Read in the new displacement by clicking on 	               .

# Solve

- Back in the System-sheet, solve the equation system by clicking on                .         .
The load-displacement curve will be updated accordingly.

- Repeat the last 3 steps for more incremental steps on the path of equilibrium.

7.3 Arc Length Method

Hint: In the Nonlinear Truss Program, a secant is used instead of a tangent for the specification of the direction of the predictor. Therefore, it is necessary to do a first iteration-step with the “load control method” in order to obtain 2 points from the equilibrium path to determine the direction of the secant.

Procedure:

In the System-sheet:

# Sol. options

- Check "arc length method" in the 		              of the System-sheet.

- Check also „constraints“ in the column of „Display“.

# OK

- Confirm your choices with 		.

# Predictor

- Click  	          to get to the Predictor-sheet.

- An equation system appears in matrix notation.

In the column C the current displacements and the current load factor are displayed. Column D shows the distance of the next point of equilibrium in the direction of the displacements and the load vector respectively, which will be reached with the next predictor.

# 1.0

Change the length of the predictor (column H) e.g.
Note that the length of the new predictor is always calculated relatively to the length of the last increment (the length of the last secant).

# Read

- The new value has to be read in.

# Solve

- Back in the System-sheet, solve the equation system by clicking on 	         .
The load-displacement curve will be updated accordingly.

- Repeat the last 3 steps to change the length of the predictor again and calculate more steps on the equilibrium path.

9. Special features

- If the „arc length method“ is active the equilibrium path can be generated automatically:

Click on 	               and enter the number of incremental steps which should be done.

# Automatic

Note that the length of the predictor will be constant for these incremental steps.

# Reset

- You can go back in your calculation to any previous step using the 	    -button:	
 	for the „load control method“ in the Loads-sheet
 	for the "displacement control method" in the Nodes-sheet
 	and for the "arc length method" in the Predictor-sheet sheet

# Bracketing

- When the "arc length method" is selected you can find critical points (det K = 0) by clicking on
and entering a giving amount of steps, if you are next to such critical point.

Types of critical points:

- bifurcation point (detK = 0)

- limit point (detK = 0 and horizontal tangent of the equilibrium paths )

- turning point (detK = 0 und vertikal tangent)  not in this example

- How to get secondary path in this diagram?

Set the “Combi” factor in the Predictor-sheet to 0.The search direction of the predictor will be changed from the primary path to the secondary path.

# Read

  - The new input again has to be read in by clicking

  - Continue solving the problem as described in section 7.3.

10. Export

You can export your calculation example into an .xlsx file (EXCEL® file without macros) which contains all your input like nodal coordinates, elements and loads – and also solution information if you already solved your problem. The exported .xlsx file is only a data storage. In order to run an exported file, look at chapter 11. Import.

11. Import

You can import the data base of an exported example by clicking on Import in the nodes sheet and selecting the respective .xlsx file.

# new

If you just want to use the imported data base a base for your new calculation, click on                          on the nodes sheet and proceed with steps 1-9.

# Restart

If you want to restart from the results of your exported file, or just want to visualize the results, click                         and select take results of previous run.

# Read

Then you have to confirm the node, elements and loads by clicking                          at the respective sheets. Now you are able to visualize the results of the previous run in the movie sheet, or to continue your non-linear calculation.

12. Remarks

# Refresh

If you encounter problems with the movie sheet, try the                       button on the movie sheet.

# new

Sometimes also clicking                         on the nodes sheet can help.

Please let us know if you encounter any errors / bugs, so that we can fix them as soon as possible.

Last modification: 22.04.2016, ag

Last modification: 20.04.2020, kub